SMART AIR-SHIELD -A Helmet-Mounted Air-Quality Monitoring and Particulate-Exposure Reduction Attachment for Two-Wheeler Riders

Theme: Sustainable Cities / Smart Mobility (Hardware Innovation)

Team Requirement:

Student 1 – Mechanical: CAD + helmet mounting + 3D printing
Student 2 – ECE/EEE: Sensors + electronics + battery + blower control
Student 3 – CSE/ECE: ESP32 programming + data logging + mobile/dashboard

Abstract

Two-wheeler riders are directly exposed to rapidly changing roadside and industrial particulate pollution during travel. This project proposes SMART AIR-SHIELD, a compact removable helmet attachment that combines hyperlocal PM2.5/PM10 monitoring with adaptive filtered-air delivery to the rider’s breathing zone. A particulate sensor measures the surrounding exposure environment while a variable-speed centrifugal blower draws air through a replaceable particulate filter and delivers the treated air through a short duct near the nose–mouth region. The controller adapts blower speed to particulate concentration and records exposure data for route-wise analysis. The prototype will be evaluated through bench airflow/pressure-drop tests, controlled particle tests and paired on-road measurements with the system OFF and ON. Performance will be quantified using particulate reduction, airflow, pressure drop, energy consumption, battery runtime, noise and comfort. A key design requirement is that the attachment be removable and non-structural, without drilling or cutting the protective helmet shell. The project aims to provide an affordable personal exposure-reduction technology for two-wheeler riders while generating high-resolution local air-quality data useful for identifying pollution hotspots

Problem Statement

Two-wheeler riders in Indian cities face continuous exposure to PM2.5/PM10, CO, and VOCs during daily commutes. Conventional helmets protect against impact but do not monitor air quality or actively reduce inhaled particulates, leading to long-term respiratory and cardiovascular risks. There is a need for a low-cost, retrofit solution that measures in-helmet pollution and adaptively reduces exposure without compromising comfort or safety.

Proposed Solution

SMART AIR-SHIELD is a modular external accessory attached to a full-face helmet using a clip-on/strap-assisted mount. The system works like a small personal air-treatment loop. The ambient sensor measures the pollution level outside the helmet. A blower draws air through a particulate filter. The filtered stream travels through a short flexible duct and is released near the nose–mouth region. A second sampling point near the outlet can be used to measure the delivered-air particulate concentration.

Operating sequence

Step 1 – Ambient air enters the module

A protected inlet on the external module samples the surrounding air. The inlet should face the environment but should not be positioned directly behind the rider’s exhaust plume.

Step 2 – Pollution is measured

A PM2.5/PM10 optical particle sensor measures particulate concentration. This gives the controller a real-time indication of the rider’s surrounding particulate exposure.

Step 3 – The blower draws air

A compact centrifugal blower creates the pressure difference needed to move air through the filter. A centrifugal blower is preferable to a simple low-pressure fan because the filter and duct introduce flow resistance.

Step 4 – Air passes through the filter

A replaceable filter cartridge contains a coarse pre-filter followed by a fine particulate filter. An optional activated-carbon layer may be included for some odour/volatile-organic-compound reduction, but the prototype should not claim complete removal of exhaust gases.

Step 5 – Filtered air travels through the duct

A small flexible duct, approximately 8–12 mm internal diameter as an initial design range, follows the lower/side edge of the helmet and terminates near the chin/nose–mouth region.

Step 6 – Filtered air is delivered to the breathing zone

The outlet should create a gentle local flow around the nose–mouth area rather than sealing the face. The objective is to reduce the particulate concentration in the air immediately available for breathing.

Step 7 – The second sensor checks the outlet

A second PM sampling point can be placed near the delivered-air region. This allows the team to compare ambient and breathing-zone particulate concentration.

Step 8 – The controller adapts the blower

When particulate concentration increases, the controller can increase blower speed. Under cleaner conditions it can reduce speed to save battery energy and reduce noise.

Basic working concept

Ambient polluted air → PM sensor → filter + centrifugal blower → flexible duct → breathing zone near nose/mouth → outlet PM measurement → controller adjusts blower speed

Components Required for the First Prototype

| Subsystem | Suggested component | Purpose | Approx. prototype range |
| --- | --- | --- | --- |
| Helmet | BIS-marked full-face helmet | Base platform for the removable attachment | ₹1,500–3,500 |
| PM sensing | PM2.5/PM10 optical sensor(s) | Ambient and/or outlet measurement | ₹2,000–6,000 |
| Controller | ESP32-class development board | Sensor reading, fan control, data logging/Bluetooth/Wi-Fi | ₹400–1,000 |
| Blower | Compact variable-speed centrifugal blower | Moves air through filter and duct | ₹800–2,000 |
| Filter | Coarse + fine particulate filter cartridge | Particle removal | ₹500–1,500 |
| Battery | Protected Li-ion/LiPo pack with BMS | Portable power | ₹1,000–2,500 |
| Driver | MOSFET/PWM blower driver | Variable-speed control | ₹150–400 |
| Duct | Flexible silicone/TPU tube | Filtered-air delivery | ₹200–600 |
| Enclosure | 3D-printed ABS/PETG/nylon-type enclosure | Protect electronics and filter/blower | ₹500–1,500 |
| Display/indicators | OLED/LED/buzzer | Status indication | ₹150–500 |
| Mount | Soft-lined clips/strap bracket | Removable attachment | ₹300–800 |
| Miscellaneous | Wires, connectors, PCB, fuse, switch, fasteners | Assembly and protection | ₹800–1,500 |